http://www.freesoft.org/CIE/RFC/1042/6.htm

### Frame Format and MAC Level Issues

#### For all hardware types

IP datagrams and ARP requests and replies are transmitted in standard 802.2 LLC Type 1 Unnumbered Information format, control code 3, with the DSAP and the SSAP fields of the 802.2 header set to 170, the assigned global SAP value for SNAP [6]. The 24-bit Organization Code in the SNAP is zero, and the remaining 16 bits are the EtherType from Assigned Numbers [7] (IP = 2048, ARP = 2054).

IEEE 802 packets may have a minimum size restriction. When necessary, the data field should be padded (with octets of zero) to meet the IEEE 802 minimum frame size requirements. This padding is not part of the IP datagram and is not included in the total length field of the IP header.

For compatibility (and common sense) the minimum packet size used with IP datagrams is 28 octets, which is 20 (minimum IP header) + 8 (LLC+SNAP header) = 28 octets (not including the MAC header).

The minimum packet size used with ARP is 24 octets, which is 20 (ARP with 2 octet hardware addresses and 4 octet protocol addresses) + 8 (LLC+SNAP header) = 24 octets (not including the MAC header).

In typical situations, the packet size used with ARP is 32 octets, which is 28 (ARP with 6 octet hardware addresses and 4 octet protocol addresses) + 8 (LLC+SNAP header) = 32 octets (not including the MAC header).

IEEE 802 packets may have a maximum size restriction. Implementations are encouraged to support full-length packets.

For compatibility purposes, the maximum packet size used with IP datagrams or ARP requests and replies must be consistent on a particular network.

Gateway implementations must be prepared to accept full-length packets and fragment them when necessary.

Host implementations should be prepared to accept full-length packets, however hosts must not send datagrams longer than 576 octets unless they have explicit knowledge that the destination is prepared to accept them. A host may communicate its size preference in TCP based applications via the TCP Maximum Segment Size option [10].

Datagrams on IEEE 802 networks may be longer than the general Internet default maximum packet size of 576 octets. Hosts connected to an IEEE 802 network should keep this in mind when sending datagrams to hosts not on the same IEEE 802 network. It may be appropriate to send smaller datagrams to avoid unnecessary fragmentation at intermediate gateways. Please see [10] for further information.

- IEEE 802.2 Details

- IEEE 802.3 Details

- IEEE 802.4 Details

- IEEE 802.5 Details

http://www.erg.abdn.ac.uk/users/gorry/course/inet-pages/arp.html

# Address Resolution Protocol (arp)

The address resolution protocol is a protocol used by the Internet Protocol (IP) network layer protocol to map IP network addresses to the hardware addresses used by a data link protocol. The protocol operates below the network layer as a part of the OSI link layer, and is used when IP is used over Ethernet.

The term address resolution refers to the process of finding an address of a computer in a network. The address is "resolved" using a protocol in which a piece of information is sent by a client process executing on the local computer to a server process executing on a remote computer [ this guy writes too technical]. The information received by the server allows the server to uniquely identify the network system for which the address was required and therefore to provide the required address. The address resolution procedure is completed when the client receives a response from the server containing the required address.

An Ethernet network uses two hardware addresses which identify the source and destination of each frame sent by the Ethernet. The destination address (all 1's) may also identify a broadcast packet (to be sent to all connected computers). The hardware address is also known as the Medium Access Control (MAC) address, in reference to the standards which define Ethernet. Each computer network interface card is allocated a globally unique 6 byte address when the factory manufactures the card (stored in a PROM). This is the normal source address used by an interface. A computer sends all packets which it creates with its own hardware source address, and receives all packets which match its hardware address or the broadcast address. When configured to use multicast, a selection of multicast hardware addresses may also be received.

The Ethernet address is a link layer address and is dependent on the interface card which is used. IP operates at the network layer and is not concerned with the network addresses of individual nodes which are to be used. A protocol known as the address resolution protocol (arp) is therefore used to translate between the two types of address. The arp client and server processes operate on all computers using IP over Ethernet. The processes are normally implemented as part of the software driver which drives the network interface card.

There are four types of arp messages which may be sent by the arp protocol. These are identified by four values in the "operation" field of an arp message. The types of message are:

- ARP request

- ARP reply

- RARP request

- RARP reply

The format of an arp message is shown below:

Format of an arp message used to resolve the remote MAC Hardware Address (HA)

To reduce the number of address resolution requests, a client normally caches resolved addresses for a (short) period of time. The arp cache is of a finite size, and would become full of incomplete and obsolete entries for computers that are not in use if it was allowed to grow without check. The arp cache is therefore periodically flushed of all entries. This deletes unused entries and frees space in the cache. It also removes any unsuccessful attempts to contact computers which are not currently running.

### Example of the use of the Address Resolution Protocol (arp)

The figure below shows the use of arp when a computer tries to contact a remote computer on the same LAN (known as "sysa") using the "ping" program. It is assumed that no previous IP datagrams have been received form this computer, and therefore arp must first be used to identify the MAC address of the remote computer.

The arp request message ("who is X.X.X.X tell Y.Y.Y.Y", where X.X.X.X and Y.Y.Y.Y are IP addresses) is sent using the Ethernet broadcast address, and an Ethernet protocol type of value 0x806. Since it is broadcast, it is received by all systems in the same collision domain (LAN). This is ensures that is the target of the query is connected to the network, it will receive a copy of the query. Only this system responds. The other systems discard the packet silently.

The target system forms an arp response ("X.X.X.X is hh:hh:hh:hh:hh:hh", where hh:hh:hh:hh:hh:hh is the Ethernet source address of the computer with the IP address of X.X.X.X). This packet is unicast to the address of the computer sending the query (in this case Y.Y.Y.Y). Since the original request also included the hardware address (Ethernet source address) of the requesting computer, this is already known, and doesn't require another arp message to find this out.

http://www.inetdaemon.com/tutorials/lan/arp.html

Address Resolution Protocol

RFC 826 defines Address Resolution Protocol (ARP). ARP is the means by which an IP address is translated into a physical MAC address. By using ARP, a packet destined for a logical IP address may be delivered to a computer with a Network Interface Card (NIC) with a specific hardware address. All network hosts maintain their own ARP tables which is usually formatted as follows:

ARP is used to build a host table listing the network protocol, the protocol's logical address, and the physical address (MAC) of that host. All hosts in a broadcast domain will passively listen to broadcast ARP packets, and will record information heard in these broadcast packets to its host table. A host will actively attempt to discover a particular address only when it looks for a logical address it does not have in its host table. A simple ARP host table might look something like this:

| Protocol | Protocol Address | Hardware Address (MAC) |
| --- | --- | --- |
| IP | 206.1.32.5 | BA D0 BE EF FF FA |
| IP | 206.1.32.12 | BA D0 BE EF FF C2 |
| IPX | AF D3 4A 6C BA D0 BE EF | BA D0 BE EF FF AB |

ARP PACKET STRUCTURE

TRANSMISSION LAYER for 802.x protocols

1       2       3       3       4

0      7       5       3       1       9       7

+------------------------------------------------+

|          Destination MAC Address               |

+------------------------------------------------+

|            Source MAC Address                  |

+---------------+--------------------------------+

| Protocol Type |

+-------+-------+

ARP

PACKET                 1       2       3       3       4

DATA    0      7       5       3       1       9       7

+---------------+---------------+----------------+

|      HRD      |      PRO      |  HLN  |   PLN  |

+---------------+---------------+----------------+

|      OP       |      SHA ...

+------------------------------------------------+

|      SPA ...

+------------------------------------------------+

|      THA ...

+------------------------------------------------+

|      TPA ...

+------------------------------------------------+

|      SHA ...

Ethernet transmission layer

(not necessarily accessible to the user):

DESTINATION     48.bit: Destination Address

SOURCE          48.bit: Source Address

PROTOCOL TYPE   16.bit: Protocol type

(set to ARP).

Ethernet packet data:

HRD   16.bit: Hardware address space

(e.g., Ethernet, Packet Radio Net.)

PRO   16.bit: Protocol address space.

For Ethernet hardware, this is from

the set of type fields ether_typ$.

HLN    8.bit: Hardware Address Length (0-255 Bytes)

PLN    8.bit: Protocol Address Length (0-255 Bytes)

OP    16.bit: Opcode: either request or reply

SHA   nbytes: Sender Hardware Address (this packet),

n from the HLN field.

SPA   mbytes: Sender Protocol Address (this packet),

m from the PLN field.

THA   nbytes: Target Hardware Address (this packet),

(if known).

TPA   mbytes: Target Protocol Address

ARP ADDRESS RESOLUTION PROCESS

- A routing protocol determines the next-hop IP address.

- Is the next hop MAC for the IP in question in the IP-ARP table?

  - YES: create an 802.x packet using the destination MAC and send.

  - NO:

    - Generate Ether packet with:

| DESTINATION | = | Unset |
| --- | --- | --- |
| SOURCE | = | LOCAL MAC |
| Protocol type | = | Address Resolution |
| HRD | = | local MAC |
| PRO | = | IP |
| HLN | = | 6 |
| PLN | = | 4 (if IP) |
| OP | = | REQUEST |
| SHA | = | local MAC (Requestor) |
| SPA | = | local IP (Requestor) |
| THA | = | (broadcast hardware address) |
| TPA | = | remote IP |

    - Broadcast ARP packet to the wire.

- Next-Hop Host will update it's IP-ARP table with the requestor's IP address and MAC, then respond with packet set to:

| DESTINATION | = | Unset |
| --- | --- | --- |
| SOURCE | = | LOCAL MAC |
| Protocol type | = | Address Resolution |
| HRD | = | local MAC |
| PRO | = | IP |
| HLN | = | 6 |
| PLN | = | 4 (if IP) |
| OP | = | REPLY |
| SHA | = | local MAC |
| SPA | = | local IP |
| THA | = | requestor's MAC |
| TPA | = | requestor's IP |

- OTHER Hosts:
Any other host will update it's IP-ARP table with the requestor's info, then drop the packet.

http://students.cs.byu.edu/~cs460ta/cs460/labs/arp.html

# ARP Lab

Background:

The Address Resolution Protocol is used to determine the Ethernet address of a host given the IP address.  For our experiments, the ARP code is a separate object that sits between the Ethernet layer and the internet layer.  The push method on the ARP object will be passed the IP address of the desired destination.  The ARP code should then look up that IP address in the ARP table to see if there is a corresponding Ethernet address.  If so, that Ethernet address will be used to push the same payload to the Ethernet layer.  If the IP address is not found, then the ARP object will create an ARP packet, send it out on the broadcast Ethernet address and wait for a reply.  The host with the corresponding IP address should reply with an ARP packet.  If there is no response within 10 seconds, then the ARP push routine should print an error message and return.

The ARP packet is contained directly in an Ethernet packet and does not have an IP header.  In order to recognize the ARP packet, you need to look at the type field in the Ethernet packet.  The type field should be 0x806 for ARP packets.  Because Ethernet assigns a single type value to ARP, an Ethernet frame that contains an ARP request message will have the same type as an Ethernet frame that carries and ARP response.  The receiver must examine the OPERATION field in the ARP packet to determine whether an incoming message is a request or a response.

This list of current Ethernet types  may be interesting to you.  The most important numbers are:

- 0-0x5dc are reserved for IEEE802.3 Length Field.  If the type is in this range, it is interpreted as an 802.3 packet with the length in the type field.  Since Ethernet packets are limited to 1500 bytes, the reserved values can identify any valid length for a packet.

- 0x806 is the ARP type.

We will provide you with physical and ethernet layers that will allow you to talk directly to the real ethernet in the lab.  The ethernet layer will interpret the length parameter as both length and ethernet type.  The top 16 bits of the length field will have the type field for the ethernet packet.  The bottom 16 bits will have the actual length to be pushed.  The same interpretation will be made for packets that are being "popped" up from the ethernet layer to your arp code.  Be careful here, because if you don't strip off the top 16 bits, it will look like a really long pop (you should "and" the length parameter with 0xffff in order to get the real length). You should also use caution in interpreting the meaning of the length parameter. It is the length of the ethernet frame, not the length of the protocol packet inside of the frame. You will need to look at the header of the protocol (ARP or IP) in order to determine the amount of data in the packet.

When sending an ARP packet, the destination address should be the broadcast Ethernet address (0xff ff ff ff ff ff).  The ARP layer should use this address when sending an ARP packet.  When the Ethernet layer sees this address, it will pass the packet up the stack, even though it does not match the hardware Ethernet address.

Specifications:

You should study the material on pages 268-271  in order to complete this lab successfully.  The format for an ARP packet is described on page 270.

Here is an example of how an ARP packet should look - http://students.cs.byu.edu/~cs460ta/cs460/labs/Arp.html.

| Hardware type = 1 |  | ProtocolType = 0x800 |
| --- | --- | --- |
| HLen=6 | PLen=4 | Operation |
| SourceHardwareAddr (bytes 0-3) |  |  |
| SourceHardwareAddr(bytes 4-5) |  | SourceProtocolAddr(bytes 0-1) |
| SourceProtocolAddr(bytes 2-3) |  | TargetHardwareAddr (bytes 0-1) |
| TargetHardwareAddr (bytes 2-5) |  |  |
| TargetProtocolAddr (bytes 0-3) |  |  |

The ARP layer object will be very similar to the physical layer object you wrote in a prior lab.  The ARP layer object must support the following methods:

- init ( char *name, protocol *higher, protocol *lower, char *addr, int port, int client); The initialization code will be called with the addr field corresponding to your machine's IP address in the bottom 32 bits and your machine's ethernet address in the top 48 bits (for a total of 10 bytes of address).  You should save this away so that you can respond to ARP requests that occur later.

- push ( char *buf, int len, char *addr);  The push method will

  - Look up the destination IP address in the ARP table.

  - If the address is found, send the payload untouched to the Ethernet layer, specifying the destination Ethernet address from the table.  You should put 0x800 in the top 16 bits (type field) of the length to indicate that this is a normal IP packet.

  - If the address is not found, send an ARP packet with the Operation=1 for REQUEST and sleep for 10 seconds (you indicate that this is an ARP request by putting the value 0x806 in the top 16 bits of the length parameter).

  - If a response to the ARP occurs before the sleep finishes (an ARP packet is received with Operation=2 for RESPONSE), the ARP table will be filled in with the correct value.

  - When the thread wakes up, it should check the ARP table again.  If the address is still not found, return, otherwise, send the packet to the destination specified in the ARP table.

- pop ( char *buf, int len);When this method is called, you should

  - See if the top 16 bits of len is 0x806 (remove these bits and use the bottom 16 bits for the real length).  If so, this is an ARP request.  You should see if the Target Protocol Address is your IP address.  If so, create an ARP response packet (swap the target and source pairs and change the OPERATION field to 2 for response).  If you are not the target, ignore it.  If you are the target, you should add the source host's IP and Ethernet address to your ARP table.

  - If the top 16 bits of len is 0x800, pass the packet untouched up to the next layer.

Template

The arp framework can be found at ~cs460ta/labs/arp_template (this path is found when using your account on the cs460 lab machines - just "cd" to this location). It contains the TA version of physical and ethernet layers that talk to the real ethernet. It also contains a template for the arp layer and the test version of internet and main. The framework will provide you with a physical layer that talks to a daemon (ether_service) that runs on each machine in the lab. You will need to use the template versions of the physical layer and ethernet layer in order to talk through the daemon to the ethernet card. The traffic you generate in this lab will be sent to the 2nd ethernet card (eth1) and you will need to select eth1 in the capture window for ethereal. Your main debugging tool will be to use ethereal to monitor traffic on this network link.

Passoff:

The passoff procedure will test the following specific items

The main() code will send a packet from the client to the destination IP address.  On the first send, you should send an ARP packet to get the destination Ethernet address, after that, packets with that target IP address will be sent directly to the correct Ethernet address.

The main() code will then send packets to different IP addresses.  Your code should send another ARP packet when each new IP address is presented, but should remember which addresses it already knows.

The server code will send ARP packets to your client with your target IP and you should respond with your Ethernet address.

Tips:

You will probably want to turn on "Update list of packets in real time" and "Automatic scrolling in live capture" in the ethereal capture window so you can see packets as they arrive.

The arguments to the binary are now different.  The argments are (host where daemon is running) (port daemon is listening to) (src IP address) (src ETH address)  (dst IP address) (dst ETH address) (-c or -s) (-v optional debug flag) (0xff debug level).  The additional arguments were included so that you can run your code without interfering with other students.  Pick IP addresses and Ethernet addresses with the same top bytes as this example, then change the bottom byte between 0 and 200.

Example: 
On the machine 460seven run 
./arp 460seven 1234 192.168.102.7 00:a0:cc:66:99:33 192.168.102.2 00:a0:cc:66:99:35 -c -v 0xff 
On the machine 460five run with exactly the same arguments, except for the hostname and client flag 
./arp 460five 1234 192.168.102.7 00:a0:cc:66:99:33 192.168.102.2 00:a0:cc:66:99:35 -s -v 0xff

Type in a test string on the client machine and it should send an arp packet to the server machine.  The IP addresses and Ethernet addresses you are supplying on the command line do not really exist so no real machines will respond to them.  There are some real IP addresses hardcoded in main.cc that will test the response of a real machine.  Your code will be answering the ARP on another machine that has the same fictional IP and Ethernet values.  When the server indicates that you have passed off that part of the lab, hit the enter key on the server and he will send back a packet to the client.

You will always connect to the daemon on port 1234.  The daemon is "ether_service" and you can use "ps -ef | grep service" to make sure it is running on  the machine you are working on.  You will probably want to run this on machines where nobody else is working so you dont have as much traffic to sort through in the sniffer.

While you are getting your code working, you will probably want to turn on some serious debug statements.  If you run the lab code with something like "./arp 460seven 1234 192.168.102.1 00:a0:cc:66:99:33 192.168.102.2 00:a0:cc:66:99:35 -c -v 0xff" it will turn on a lot of debug in the lower layers.  Bit 0 turns on the physical layer printfs.  Bit 1 (0x2) turns on the ethernet debug.  Bit 2 (0x4) turns on a printf when something is not destined for your ethernet address.  Bit 3 (0x8) turns on the basic arp debug.  Bit 4 (0x10) just shows what is being put into the arp table.

Run your code, compiled with the versions of main.cc and internet.cc found in the framework.   The lab version of the code can be run directly from ~cs460ta/labs/arp.  The client code should prompt you for a string to send.  This string will be pushed to your arp code and when the arp response is received from the server (lab version of the code) the packet should be sent.  This string should then appear on  the server window.  The client will then send the characters 0xfeedbeef and this should be displayed on the server  (lab version of the code).  The client will then send The client will then send the characters 0xfeedbeef  to another IP address and you should generate another ARP.  The lab code will respond to this other address and should print out another passoff message indicating that it has received the string.  You should then press the enter key on the server and it will send an arp request packet to your client.  If you respond correctly, the server will send 0xfeedbeef back to you and your code will display a passoff message.

You may want to run additional tests of your own to make sure that things are working.

The following passoff levels will apply to this lab:

| Passoff Level | Behavior | Points |
| --- | --- | --- |
| Minimal Passoff | At least one ARP packet is correctly sent and received and the data packet is then sent to the correct Ethernet destination. | 2 Points |
| Perfect behavior | Passes all tests | 3 Points |

http://linux.ce.sharif.edu/new/courses/79-80/2/ce443/projects/delivered/16/

ARP Attack

computer network project saeed mirzaeian 77177507

How to write a network program? What is ARP? What is ARP Attack? How do I wrote the program? Platform and language of my project. Source Code and Binary Code.

How to write a network program? The first problem in writing a network project is learning how to write it. At the begining you may prefer to write your project in Windows but you'd better not!! The main problem in writing a Windows project is that you wouldn't find any references that would help you manage it on the web. So take my advice and try to get used to the user enemy platform of Linux or something like this. If you agree you can download one of the best references here... Try it! It won't let you down.

What is ARP? Address Resolution Protocol (ARP) is a protocol for mapping an Internet Protocol address (IP address) to a physical machine address that is recognized in the local network. For example, in IP Version 4, the most common level of IP in use today, an address is 32 bits long. In an Ethernet local area network, however, addresses for attached devices are 48 bits long. (The physical machine address is also known as a Media Access Control or MAC address.) A table, usually called the ARP cache, is used to maintain a correlation between each MAC address and its corresponding IP address. ARP provides the protocol rules for making this correlation and providing address conversion in both directions. How ARP Works When an incoming packet destined for a host machine on a particular local area network arrives at a gateway, the gateway asks the ARP program to find a physical host or MAC address that matches the IP address. The ARP program looks in the ARP cache and, if it finds the address, provides it so that the packet can be converted to the right packet length and format and sent to the machine. If no entry is found for the IP address, ARP broadcasts a request packet in a special format to all the machines on the LAN to see if one machine knows that it has that IP address associated with it. A machine that recognizes the IP address as its own returns a reply so indicating. ARP updates the ARP cache for future reference and then sends the packet to the MAC address that replied.

What is ARP Attack? Now you know what is ARP and you know what is ARP cache is. ARP attack is to send a fault MAC Address for an IP Address and make a fault entry for the destination computer and the destination computer refresh it's ARP cache to this fault MAC address so if the user of this computer wants to connect to this IP, he would fail. For example you tell the other computers that the MAC Address (12-fd-23-74-76-f4) belongs to IP (194.222.41.120).

How did I wrote the program? After one week of trying to find a good project, I decide to write ARP attack. I don't know why and don't ask!! The first thing is learning socket programming and this helps me alot. after that I should find a Linux based computer so I borrow Linux from my friends and try to install it!!! LISTEN : don't even think of install it on your own!!! I try this 3 times and the result is fdisk hard drive! So I go to my friends room (thanx to them all) that has Linux installed for 2 nights. Next problem we don't have a network to test our program on it...WHY don't we try it on one computer? we tried it but we fail, I don't know the exact reason but I think if we send an ARP packet to our computer it wouldn't be sent to network layer I JUST THINK. So we get 2 network adaptor and another computer with Windows98. This project contains 3 parts 1: Finding out our network data 2: Finding our destination computer data 3: Generating a suitable ARP packet and send it 1: This part of project is done by ioctl.h header file which contains ioctl instruction which can help us understand our network properties. this instruction has a number of constants which helps us. 2: We like to send our fault ARP packet to all the computers in our LAN, so we have to use broadcast IP address bus IT DIDN'T WORK AT ALL!!!! then we use another way we make a big loop that counts all IP addresses in our LAN and send our packet to all of them!! Funny but the only way we could do this! 3: First of all what is the basic structure of an ARP packet? We find this out using a source code we find on the web for send and receive arp packets, a book named "TCP/IP Illustrated" and a "pdf" file that i don't have it here. 0 1 2 3 4 5 6 11 Destination MAC Address; Source MAC Address. Packet Type; HW Type; Protocol Type; HW len; Pro len; ARP Operation; Sndr MAC Addr[2]. Sender MAC Address [4]; Sender IP Address; Target MAC Address. Target MAC addr; Target IP Address. Destination MAC Address: fill this part with broadcast MAC Address which is ff-ff-ff-ff-ff-ff ; Source MAC Address: fill it with our computer MAC Address. (which we get it with ioctl instruction). Packet Type: This field should show that this packet is an ARP packet , so we should use a predefined number: ETH_P_ARP 0x0806 HW Type: defined: 0x0001 Protocol Type: defined: 0x0800 HW length: defined: 6 Protocol length: defined: 4 ARP Operation: This field shows that this is an ARP request or an ARP reply.we use ARP request. Why? because an ARP request doesn't need to know the destination MAC Address but an ARP reply should!!! and we just want to use IP address so.... Sender MAC Address: Now it is the time. ATTACK!!! write the fault MAC address in this field and HACK!! Sender IP Address: The other part of trick the fault IP Address. Target MAC Address: As I said before we use ARP request so we shouldn't fill this field. Target IP Address: The IP Address of poor tricked computer! We make this IP with counting all IP's in our LAN. Now we can send it to where we want to..

Platform and language of my project I wrote my project in C++ and I use Linux RedHat 6.2 as my OS.

Source code and Binary code ooh My God!! As I said it was my second time using Linux and as you know LINUX FOR EXPERTS!! So what? do you know the difference of F8 and F5 if you don't just ask me. if you wrote your project and want to save it on a floppy then press F8 instead of F5! And you'll be ... But I have one of my friends source code try this (copyright Mr. Meysam Ghanbari and Mr. Vahid Asadi) To use this code: 1- extract this file with any thing you may have (Winzip...) 2- Open this file in Linux then if it has a star write "chmod -x arpattack.c" 3- Run this instruction too "gcc arpattack.c -o arpattack" 4- Run "arpattack" 5- Based on this program now you can see your computer and network data. 6- Now you are asked to enter fault IP address in this format 123.32.12.32, fault MAC address in this format 12:02:34:f9:a3:90 and destination MAC address (USELESS I don't know why he ask this!?) .

IT IS ENOUGH !!!