# Final Report

# EEL 4914 Electrical and Computer Engineering Design

# (Senior Design)

# Spring Semester 2003

MSX Network Interface

# Submitted by:

Bruno Barreyra

David Regal

Final Report [       ]

Topic Index:_______________

# Technical Objectives

Project goals:

- Design a network cartridge for the MSX computer

  - Optimize performance

  - Marketable product

- Develop driver for the Uzix Operating System, an Unix clone

Pedagogical goals:

- Experience the full design flow of a commercial product

  - Schematic Entry

  - Layout Design

  - Driver writing

  - Validation

- Successfully working in a team

- Improve Bruno’s crappy time management

# Study Plan

Required knowledge:

- Digital Circuits

- C programming

- Operating Systems

Tools:

- Either Protel or Eagle

- Max+Plus II

- Hitech-C Compiler for Zilog Z-80

Prototype:

- The first prototype will consist of an adapted ISA prototyping card that fits in the MSX slot. This first board will not include the network chip, but will only assure that the interfacing is correct as designed

- Once the design (schematic and PCB layout) is complete, the PCB board will be sent to fabrication and possibly assembly

- The final product will be a plastic enclosed standard MSX sized cartridge with an RJ-45 jack for network connection

Resources:

- MSX Turbo-R computer for testing

- Linux system for developing driver and cross-compiling

Fabrication:

- Outsource to commercial service

# Expected Outcome

The final product will reside on a standard-sized MSX cartridge, and will be plug-and-play. The main goal is integration with the Uzix operating system, which has full TCP/IP networking capabilities, including telnet, FTP, IRC and web-browsing. It would be possible in the future to provide extensions to the MSX’s BASIC language to program the network card, with great pedagogical possibilities.

The main limiting factor for performance in our project is CPU power. The standard MSX computer runs at a clock speed of 3.58 MHz, and, at this speed, decoding network packets is quite processor intensive. The units we will work with are the Panasonic MSX Turbo-R computers. Aside from the standard 3.58 MHz Z-80 processor, these units also have a 7.xx MHz R800 CPU. This CPU is fully compatible with the Z-80 and, at this clock speed, is actually equivalent to a 28 MHz Z-80 processor. Even then, the machine will not be a speed demon on the network. Speed will be the major deciding factor on design choices.

Although the MSX will lag in performance numbers compared to today’s powerful PC’s, for its own use the performance will be more than adequate. Most software written for the MSX is quite small, so the amount of data that will be transferred in actual use will be small. For example, the popular game “Nemesis” written by Konami occupies a total of 128KB. A previous prototype of a network card, using a different controller, achieved 3.99KB/s on a MSX Turbo-R. It would take a total of 32 seconds to download the game. In comparison, the download of the popular game for the PC platform, Unreal Tournament 2003, is a 140MB file, and takes around 13 minutes to download on a home broadband connection, assuming optimal connection speed. Some people will swear over their dead mother that Nemesis is a better game.

# Study Organization

Meetings:

We will have meetings after deadlines usually on Thursday nights.

# Supplementary Information

This project can be thought of as having:

25% Hardware design and implementation

35% Software design and implementation

15% Design Integration

15% Test and validation

10% Documentation

Engineering analysis

MaxplusII - Testing by implementing address decoding with the the Altera board.

Timing diagrams

Data acquired

Speed of the card

LSA clock cycles

Artwork

Block diagram of the project

Software driver

Results

Card works

MSX with Uzix

Compare proposed objectives and quantitative expectations with achieved

Expectations and objectives were met.  However, we would like to improve the software.

Driver is written in both C and assembly.  If we are to increase the speed, we should program only in assembly.

Also, to cut down on the number of op codes fetched, Memory Mode should be used on the CS8900 instead of I/O mode.

Problems encountered

Sauder shorts are nearly impossible to detect on the small pins of the main IC chip, CS8900, and to debug the chip, it is also hard because there are no clear sections.

High-level market analysis

Cost/profits

Hours of workl

Parts

Expected number of sales.

Employment opportunities

Networking devices

Nortel

Netgear

Cisco

Embedded applications with Linux

Reference list

Websites

CS8900 pdfs