- MSX Cartridge connector/bus expansion/extension side slot

(Giovanni R. Nunes)

Female 50 pin connector

+---+

1:/CS1          2:/CS2             1 |= =| 2

3:/CS12         4:/SLTSL             |= =|

5: RSV(N.C)     6:/RFSH              |= =|

7:/WAIT         8:/INT               |= =|

9:/M1          10:/BUSDIR            |= =|

11:/IORQ        12:/MERQ              |= =|

13:/WR          14:/RD                |= =|

15:/RESET       16: RSV(N.C)          |= =|

17: A 9         18: A15               |= =|

19: A11         20: A10               |= =|

21: A 7         22: A 6               |= =|

23: A12         24: A 8               |= =|

25: A14         26: A13               |= =|

27: A 1         28: A 0               |= =|

29: A 3         30: A 2               |= =|

31: A 5         32: A 4               |= =|

33: D 1         34: D 0               |= =|

35: D 3         36: D 2               |= =|

37: D 5         38: D 4               |= =|

39: D 7         40: D 6               |= =|

41: GND         42: CLOCK             |= =|

43: GND         44: SW1               |= =|

45: +5V         46: SW2               |= =|

47: +5V         48: +12V              |= =|

49: SOUNDIN     50: -12V           49 |= =| 50

+---+

Short description of the pins:
(from Philips VG8235 manual)

1    ROM addresses 4000-7FFF select signal

2    ROM addresses 8000-BFFF select signal

3    ROM addresses 4000-BFFF select signal (for 256k ROM)

4    Slot select signal

5    Reserved signal line - use inhibited

6    Refresh cycle signal

7    CPU's WAIT request signal

8    Interrupt request signal to CPU

9    Signal expressing CPU fetch cycle

10    This signal controls direction of external databus buffer

Cartridges are selected and L level is output from each

cartridge at data transmission time

11    I/O request signal

12    Memory request signal

13    Write timing signal

14    Read timing signal

15    System reset signal

16    Reserved signal line - use inhibited

17-32 Address bus signals

33-40 Data bus signals

41    Signal ground

42    CPU clock 3.579545MHz

43    Signal ground

44,46 For insertion/removal protect

45,47 +5V power source

48    +12V power source

49    Sound input signal (-5bdm)

50    -12V power source

Gradiente's Expert has not this standard MSX connector but a so-called BUS Expansion. These expansions are used in the first japanese MSX1 models but, perhaps because it isn't a ASCII standard was "dead". In Expert's case it is a copy of the second SLOT and was reserved for expansions but not used for "MSX industry" in Brazil. It is the same pinout of MSX cartridge but... it looks like a SCSI Harddisk connector (50 pins....)

Compare:
MSX cartridge:

2       50

(...)

1       49

The BUS Expansion:

49     1

(...)

50     2

The major use of it was for transforming Expert MSX1 machines in MSX2 models, expanding the SLOT B and putting in it the V9938, ROMS and Mapper...

The information is on courtesy of Alex Mitsio Sato directly from his Expert User's Manual... :)

The Yamaha CX-5M has (apart from a normal MSX cartridge slot) also the so-called "Extension side slot". A 60 pin expansion port. According to the manual it's pin assignment is as follows:
(http://www.rgp.nl/cx5m/conn.htm)

No. Name         I/O     No. Name         I/O     No. Name        I/O

--------------------     --------------------     -------------------

1   SOUND OUT     O      2   GND                  3   GND

4   Phase Control I      5   Y	           O      6   B-Y          O

7   C VIDEO       O      8   R-Y	   O      9   EXT CLOCK    I

10  CLOCK INT/EXT I      11  CS1	   O      12  CS2          O

13  CS12	  O      14  SLTSL         O      15  N/C          -

16  RFSH	  O      17  WAIT	   I      18  INT          I

19  MI		  O      20  NO CONNECTION -      21  IORQ         O

22  MERQ	  O      23  WR            O      24  RD           O

25  RESET	  O      26  N/C	   -      27  A9           O

28  A15		  O      29  A11	   O      30  A10          O

31  A7		  O      32  A6	           O      33  A12          O

34  A8 		  O      35  A14	   O      36  A13          O

37  A1		  O      38  A0	           O      39  A3           O

40  A2		  O      41  A5	           O      42  A4           O

43  D1           I/O     44  D8	          I/O     45  D3          I/O

46  D2           I/O     47  D5	          I/O     48  D4          I/O

49  D7 		 I/O     50  D6 	  I/O     51  GND          -

52  CLOCK	  O      53  GND	   -      54  SW1          -

55  +5V           -      56  SW2	   -      57  +5V          -

58  +12V	  -      59  SOUND IN      I      60  -12V         -