- MSX Expansion

49 47 45   5 3 1

+---------//-----+

| H H H  //H H H |

| ======//====== |

| H H H//  H H H |

+-----//---------+

50 48 46  6 4 2

(At the Computer)

50 PIN ?? at the Computer.

| Pin | Name | Dir | Description |
| --- | --- | --- | --- |
| 1 | /CS1 |  | Memory Read in addresses 4000-7FFF |
| 2 | /CS2 |  | Memory Read in addresses 8000-BFFF |
| 3 | /CS12 |  | Memory Read in addresses 4000-BFFF |
| 4 | /SLTSL |  | Low when Slot 2 (cartridge slot) is selected |
| 5 | n/c | - | Not connected. |
| 6 | /RFSH |  | Refresh signal from CPU |
| 7 | /WAIT |  | OC, Tells CPU to wait. Refresh signal is not maintained |
| 8 | /INT |  | OC, Requests a interrupt to CPU (call to addr 38h) |
| 9 | /M1 |  | CPU fetches first part of instruction from memory. |
| 10 | /BUSDIR |  | NC, was used to control the data direction. |
| 11 | /IORQ |  | I/O request signal. (Address=Port) |
| 12 | /MREQ |  | Memory request signal. (Address=Address) |
| 13 | /WR |  | Write signal (strobe) |
| 14 | /RD |  | Read signal (strobe) |
| 15 | /RESET |  | Reset |
| 16 | n/c | - | Not connected. |
| 17 | A0 |  | Address 0 |
| 18 | A1 |  | Address 1 |
| 19 | A2 |  | Address 2 |
| 20 | A3 |  | Address 3 |
| 21 | A4 |  | Address 4 |
| 22 | A5 |  | Address 5 |
| 23 | A6 |  | Address 6 |
| 24 | A7 |  | Address 7 |
| 25 | A8 |  | Address 8 |
| 26 | A9 |  | Address 9 |
| 27 | A10 |  | Address 10 |
| 28 | A11 |  | Address 11 |
| 29 | A12 |  | Address 12 |
| 30 | A13 |  | Address 13 |
| 31 | A14 |  | Address 14 |
| 32 | A15 |  | Address 15 |
| 33 | D0 |  | Data 0 |
| 34 | D1 |  | Data 1 |
| 35 | D2 |  | Data 2 |
| 36 | D3 |  | Data 3 |
| 37 | D4 |  | Data 4 |
| 38 | D5 |  | Data 5 |
| 39 | D6 |  | Data 6 |
| 40 | D7 |  | Data 7 |
| 41 | GND |  | Ground |
| 42 | CLOCK |  | CPU clock, 3.579 MHz |
| 43 | GND |  | Ground |
| 44 | SW1 | - | NC, Insert/remove detection for protection |
| 45 | +5V |  | +5 VDC (300mA max /slot) |
| 46 | SW2 | - | NC, Insert/remove detection for protection |
| 47 | +5V |  | +5 VDC (300mA max /slot) |
| 48 | +12V |  | +12 VDC (50mA max /slot) |
| 49 | SOUNDIN |  | Sound input (-5dBm) |
| 50 | -12V |  | -12 VDC (50mA max /slot) |

Note: Direction is Computer relative Peripheral.