Panasonic FS-A1ST (an MSX Turbo-R)

1.1 Make sure you plug your MSX to the right voltage!

The Philis NMS8245 is an european model. Works at 220 volts. The Panasonic FS-A1ST is japanese and works at 100 volts. In case you don't know the right voltage for your MSX, just look under it. There should be a plate or a sticker telling you the right voltage. In the upper right picture you can see this sticker on the Philips NMS8245, and in the center picture the plate on a Panasonic FS-A1ST can be seen. In the upper part of the picture on ther right you can see the power plug of the FS-A1ST, and below it the power plug of the NMS8245. Also, adapters for both types of plugs are shown.

In order to plug a japanese computer (100 volts) to an european plug (220 volts mostly everywhere), or an european computer to a japanese plug, we need to use a transformer. In the picture on the right a typical european one can be seen, and in the picture on the left you can see a japanese one.

1.3 The Panasonic FS-A1ST

The Panasonic FS-A1ST is a MSX Turbo-R machine, equipped with 256Kb RAM, 128Kb VRAM and a double-sided disk drive. Comes with two CPUs: a Z80A at 3.58Mhz and a R800 at 7.16Mhz. This FS-A1ST has been modified and has a total of 512Kb RAM instead of the default 256Kb. We'll see the rest of differences between MSX generations later. The FS-A1ST is a japanese computer, so it works at 100 volts. More about this computer:

The front pannel of this computer is filled by LEDs, buttons and switches. From right to left: FDD LED, R800 mode LED (turned on when the computer is in R800 mode, and off when the computer is in Z80 mode), the PAUSE LED (some MSX computers have a PAUSE key that completely freezes the computer when pressed, and unfreezes it when pressed again, the FS-A1ST has such a key), the KANA LED (turned on when the keyboard is in Japanese mode), the CAPS LED and the POWER LED.
    Next there's a hole in the pannel. This is the internal microphone. MSX Turbo-R computers come equipped with a digital audio PCM channel, and using this microphone they can record sounds. An external microphone can also be used. We'll see this soon.
    Next there's the RESET button, followed by a small switch to turn ON or OFF the internal FS-A1ST ROM program. If we move this switch to the left, the next time we boot the FS-A1ST a ROM program will be executed instead of MSX-BASIC. This program comes with a text processor, calendar, calculator, phone guide, etc. If the switch is on the right, the computer will start MSX-BASIC (or MSX-DOS/MSX-DOS2) on boot.
    And last, there's the RENSHA CONTROLLED and its LED. This is just an autofire for games. When the RENSHA CONTROLLER is on the left the spacebar works as usual. But if the RENSHA CONTROLLER is slided to the right the autofire will turn on, and its LED will start flashing. If we keep the spacebar pressed it will fire at the same speed as the RENSHA LED.

On the left side of the computer there's the ON/OFF switch, and on the right side there's the disk drive and the two joystick ports. There's also a cartridge slot on the top of the cover, just like in the NMS8245.

In the back of the computer there are the other connectors, the second cartridge slot and the power wire. The connectors, from left to right, are: SuperVHS and RGB, RCA audio (mono) and video, RF (with a small switch to change frequencies), the printer port and above it a 3mm jack plug for the external microphone.

This time we'll use a RCA cable instead of SCART, because our TV set doesn't have a SVHS connector, and we don't have a DIN8-SCART cable. Plug it to the power socket (using a transformer and adapter if needed), and the computer is ready for work.

Chapter 2: booting and customizing

In this chapter we'll show you how an MSX computer boots. We'll learn a few MSX-BASIC commands used for customizing the colors and look of the user interface. You won't need a disk drive yet, but if you don't have one you should start looking for one now, because you'll need it for the next chapter. Good places where you can start your search are auction sites like iBazar, eBay or Yahoo! Auctions. And you can use our ads section.

2.1 Booting

Make sure that your MSX is plugged to the power and to the TV set (or monitor). Then make sure there's no disk in the disk drive and turn on the computer. If everything is plugged correctly, the MSX logo should appear in the screen.
    Below the MSX logo you can see the amount of RAM of the computer and the amount of VRAM. In this case we have 128Kb RAM and 128Kb VRAM. An MSX2 computer has a minimum of 64Kb RAM and a minimum of 64Kb VRAM, but models with less than 128Kb VRAM are very rare. Models with more than 128Kb VRAM are also very rare, but we'll explain why later. The amount of RAM displayed here may not be the real amount of RAM in your computer. In several MSX computers the logo screen will only show the amount of RAM in the primary mapper. Also, there are several machines where you can only see the amount of RAM, and not the VRAM. Don't worry about that.
    The MSX logo appears as black and white over a blue background. Only the memory information is displayed with the logo. But more things could appear on the screen if the user configured the system to do so. This will be explained in following chapters.

2.2 The interface

It will take about twelve seconds for the MSX to boot (five if your MSX isn't equipped with a disk drive). When the computer boots you'll see a screen like this one. That's the MSX-BASIC interpreter screen. Don't worry about the colors being different from the ones in the image, or if the screen doesn't look the same at all. You should care about the information you can see on the screen by now. From top to bottom: MSX-BASIC version number and its copyright, free memory for MSX-BASIC programs, and the DISK-BASIC version number. Then there's the prompt (usually 'Ok', but it could be anything else up to six characters). Then there's a character-sized box. And in the bottom of the screen there may be five words. Don't worry if they're not there.
    You can find out what generation of MSX you have by looking at the MSX-BASIC version number. It will be 1.x for an MSX1, 2.x for an MSX2, 3.x for an MSX2+ and 4.x for an MSX Turbo-R. The image over this lines was taken from an MSX2+. Then there's the copyright line. It's just there to point out that this MSX-BASIC interpreter was developed by Microsoft.
    The next line is the version number of the disk controller's ROM and DISK-BASIC (MSX-BASIC extension for managing disk drives). Forget about the version number by now. As long as this line is displayed your disk drive should be working fine. If this line isn't displayed, either your MSX doesn't have a disk drive, or the DISK-ROM has been disabled on boot time. If your MSX does have a disk drive and you can't find the DISK-BASIC line then reset the MSX (use the RESET button) and don't touch the keyboard while it boots (we'll explain this later). The DISK-BASIC line should appear now, unless your computer is broken.
    The next line (usually Ok) is the prompt. It means that the MSX-BASIC interpreter is ready to receive commands. As said before, this word could be different. Later on this chapter you'll see how to change it.
    And then there's the cursor. The cursor is always positioned where the next typed character will appear. Try typing something. The cursor will move as you type. Useful, isn't it?
    In the bottom of the screen there's the function key line. Take a look at your keyboard (upper left side). There should be five keys labeled F1 to F5 and F6 to F10. These keys are the function keys. As said before, maybe this line of the screen is not displayed yet. Try pressing the function keys on the keyboard. Each of them contains a word, in the same order as the words in the function key line. That is, the function key line shows the content assigned to each function key. The keys F6 to F10 are accessed by pressing SHIFT together with F1 to F5. Try using F6 to F10. Don't worry if the MSX beeps and displays an error message.

2.3 Changing the screen mode

When the MSX boots you'll see one of the three screens over this lines. Colors may not be the same. The type of the screen you'll see depends on the MSX you own: whether it is an MSX2, an MSX2+ or an MSX Turbo-R, or whether it is an european or japanese computer. And also it depends on the settings that made whoever owned this computer before you. We'll show you how to change these settings later. The image on the left is a 80-column display, the one in the center is a 40-column display and the one on the right is 32 columns. The first two are pure text modes, while the one in the right is a semi-graphic mode. We'll explain the screen modes of the MSX in following chapters. The most used screen mode is the 80-column text mode. Now we'll explain how to change the screen mode.

The MSX-BASIC command used for changing the screen mode is SCREEN (easy to remember, right?). If your MSX is in the semi-graphic mode, then type SCREEN 0 to change to a text mode (you have to press the ENTER key after you type every command for the MSX-BASIC interpreter to execute it). After you do so, the screen will be cleared and the new screen mode will be active. The MSX-BASIC interpreter will be again waiting for a command.

Use SCREEN 1 to switch to the semi-graphic mode. Type SCREEN 0 again to return to the text mode when you're ready to continue.

2.4 Changing the screen width

Or the number of columns of the screen. As with the SCREEN command, this one is also very intuitive: WIDTH is the MSX-BASIC command used to change the width of the screen.
    The syntax is very easy: just use WIDTH x, where x is the number of text columns you want in the screen. The minimum is one column, and the maximum in text mode is 80 columns. In the semi-graphic mode, the maximum number of columns is 32. When you specify up to 40 columns the font will look like the one in the image on the right. If the number of columns is higher than 40, then the font will look like the image in the left.

    Switch back to 80 columns now. This is the mode we'll be using from now on.

2.5 Changing the prompt

The default prompt on MSX computers is Ok. The instruction used to change the MSX-BASIC interpreter's prompt is SET PROMPT. Like the other commands we've already seen, it takes a parameter. The syntax is SET PROMPT "prompt", where prompt is the text you want the MSX to display when it's ready to accept commands. In the omage in the left we changed the prompt to "MSX". The prompt can be up to six characters long. If you try to set a longer prompt, it will be truncated to six characters, as in the image on the right. Please note that you have to enclose the new prompt in quotation marks, because it is a string instead of a numeric value. More about this in following chapters.
    You should now try to change the prompt a few times. Choose a prompt you like (or set the default prompt again (Ok) if you wish), and let's continue.

2.6 Turning ON/OFF the function key line

As said previously, the display of the contents of the function keys can be turned ON and OFF. Experienced users often turn it OFF because they can remember the contents of each key, and this frees one more line in the screen for listings. But in is up to you to to leave it ON or OFF. We'll explain how.
    The command used for turning ON and OFF the function key line is KEY. The syntax is soooo simple: KEY ON turns the line on, and (you guessed) KEY OFF turns it OFF.
    In the image on the left you can see the results of a KEY OFF command, and in the image in the right the result of a KEY ON. Leave it as you like it. Moving to the next part...

2.7 Changing colors

We're now going to learn how to change the colors of the foreground (text color) and the background. The default is dark blue for the background and white for the text. But there are more colors that you can choose from. In the image on the left there is the default color palette of an MSX computer. This palette can be altered, but this will be explained in other chapter. The first two colors (0 and 1) are transparent and black.
    The MSX-BASIC instruction used to change the colors on the screen is COLOR. Up to three parameters can be specied (they must be comma-separated). The first parameter is the foreground color. The second one (if entered) is the background color. And the third one (if entered) is the border color. In text modes, the background is always the same color as the background. Take a look at the image in the center. We typed COLOR 15,2 to change to a white text (color 15) over a green background (color 2). We didn't specify a border colour, because it is senseless in text mode. Try changing colors in semi-graphic mode and experiment a bit. In the image in the right we changed again the colors, this time to a yellow text over red background.

    Try a few color combinations. Choose the one that you like the most and continue reading.

2.8 Saving changes

Not all the changes we made were permanent. After a RESET or reboot only the prompt will remain as you left it. The command used to save the changes to the battery-powered memory is SET SCREEN. After a SET SCREEN all the changes you made will be restored on each boot of the MSX computer. The information will be saved to the clock chip (which we'll explain in following chapters).

Summary

This are the MSX-BASIC commands and instructions we learned in this chapter:

- * SCREEN

- WIDTH

- SET PROMPT

- * KEY

- COLOR

- SET SCREEN

The ones marked with an asterisk have more parameters that we didn't explain in this chapter.

    Now you should be able to:

- Change the screen mode

- Change the screen width

- Change the prompt

- Turn ON and OFF the function keys line

- Change the foreground and background colors

- Make these changes permanent

Don't forget to write all your questions or comments to howto@aamsx.org.

Chapter 3: Using diskettes

In this chapter we'll see how to use diskettes on an MSX computer. Most of the disk drives found on MSX computers are 3.5 inches, single or double-sided. But also a few 5.25 inches drives can be found sometimes. Talent and Spectravideo are brands that built 5.25 inch drives. The diskettes in the pictures are 3.5 inches. The one in the picture in the left is a high-density disk, and the one in the right is a double-density disk. They can be recognized because of the extra hole in the high-density disks. Diskettes can be single or double sided, and single, double or high density. Single density disks are no longer used, so we won't talk about them. But they could be used in an MSX. We'll only talk about 3.5 inch diskettes from now on, but most of what we'll see also applies to 5.25 inch diskettes.

    These are the different disk formats available (not including single-density disks):

| Size | Sides | Density | Capacity |
| --- | --- | --- | --- |
| 5.25 inch | Single sided | Double density | 320Kb |
| 5.25 inch | Double sided | Double density | 640Kb |
| 5.25 inch | Double sided | High density | 1280Kb |
| 3.5 inch | Single sided | Double density | 360Kb |
| 3.5 inch | Double sided | Double density | 720Kb |
| 3.5 inch | Double sided | High density | 1440Kb |

High density disks cannot be used on MSX computers. A few technical documents about the MSX Turbo-R disk controller suggests that high-density disks are supported by the controller, but high-density support is not implemented in the MSX BIOS (more about the BIOS later).

3.1 Write protecting the disks

Floppy disks can be easily write-protected. When a disk is write protected it cannot be altered: files cannot be written, altered or deleted. For the rest of the chapter you must leave the disk unprotected. This is how you should do this:
    Look at the back of the diskette. There's a small hole in the right side of it, and if it's a high-density disk there will be another hole on the left. The hole on the right is used for protecting the disk. Inside of this hole there should be a small piece of plastic that can be moved using a pointy object (like a pencil). When this piece is blocking the hole the disk is unprotected and can be written to. So, if the disk is protected, unprotect it now and let's continue.

3.2 Formatting disks

Before using disks we'll have to format them. Formatting a disk means to create the sectors where the information will be stored, and setting up a pair of tables where the information about the files in the disk will be saved. The command used in MSX-BASIC to format a diskette is CALL FORMAT, without parameters.

    IMPORTANT: Please make sure that you're using a blank diskette from now on, or one that contains no important files. What we'll show now will destroy the contents of the disk.

Insert the diskette in the disk drive and type CALL FORMAT and ENTER. The MSX will ask you about which drive to format (A or B) and whether you want to format it single or double-sided. If you didn't press the CTRL key while the MSX was booting, the MSX will always ask you for what drive to format, even if your MSX has only one disk drive. This is because the MSX BIOS emulates a second disk drive, what's useful when you want to copy files from one disk to another. Holding the CTRL key while the MSX boots will turn off this emulation (we'll explain later why should/shouldn't we do this).
    The picture in the right shows the MSX-BASIC asking how it should format the disk. First it will ask about the drive name where the disk is inserted (unless you did press CTRL while booting). Just press 'A', so we'll format the disk in the A: drive. Then it will ask you about the number of sides of the formatted disk. Choose '2' if available in your MSX (for double-sided). Then the MSX-BASIC will ask you to press a key to start formatting. When you press a key, the FDD led will go ON and after a few seconds the prompt will appear again. The disk is now formatted and ready to use.

3.3 The high-density hole

We formatted the disk on the MSX at double density. It will work perfectly on the MSX. But most likely you will need to use the same diskette on a PC. To do so, you should cover the hole that indicates that the disk is high-density. Use some duct tape or a small sticker to cover it. Update: Adriano Camargo Rodrigues da Cunha told us that several disk drives use optical recognition of the HD hole, so it should covered using an opaque cover. If you don't cover this hole, the disk drive on the PC will detect a high-density disk and most likely the operating system on your PC will try to read it as if it was formatted at high-density instead of double-density.

3.4 Using MSX disks on another computer

Why should you want to use MSX disks on a different computer? Well, perhaps you're using a PC to access the Internet, and perhaps you're using it to download files that you want to use on the MSX. So you need to copy the downloaded files from the PC's hard disk to the MSX disk. You're lucky. MSX-formatted disks are 100% compatible with MS-DOS® and Windows® disks. You can just insert the MSX disk in the PC diskdrive and copy files to it as if it was a normal PC disk. And vice versa: you can copy files from an MSX disk to the PC harddisk. From now on we'll include a few files in each chapter that you will have to copy to an MSX disk.

    As an example, download this text file and copy it to the MSX disk you formatted before:

file.txt (32Kb)

If you don't know how to download files from a web site please read the documentation of your web browser.

    IMPORTANT: since PC-formatted disks can be used in the MSX, you could be tempted to format disks on the PC. Don't do that if you want to use these disks in the MSX often. When the MSX formats a disks, it writes a small program in the beginning of the disk, which is loaded each time the MSX boots with that diskette in the drive. This program in the beginning of the disk is called the boot sector, and the PC also saves its own boot sector in the disk when formatting. If you boot an MSX computer with a PC-formatted disk in the drive, most likely the MSX will hang.

3.5 Browsing the contents of the diskette

Insert the diskette in the MSX if you haven't done so yet. The MSX-BASIC command used to display the contents of a diskette is FILES. Do it. Type FILES and press ENTER. The MSX should read from the disk drive and print on the screen the name of all the files found on the disk. In this case the MSX will just print 'FILE.TXT', because that's the only file on the disk right now (the one you copied from this site before). Then the prompt appears again. In the picture on the right there's the same example, just with a diskette with a lot more files. If there were no files in the disk, the MSX-BASIC would have returned a File not found error.

    FILES can be used to find files in the disk, using parameters that we'll show in other chapter.

3.6 Displaying the free space in the disk

The MSX-BASIC function DSKF(x) is the one we'll use to display the available space on a diskette. 'x' is the drive number of the diskette we're referring to, 0 being the A: drive, 1 the B: drive and so on. A function always returns a value to the program that called it. This is why if we just type 'DSKF(0)' the MSX-BASIC interpreter will return a Syntax error. We will use the PRINT instruction to print the value returned by DSKF(). Type 'PRINT DSKF(0)' and press ENTER. The free space available in the disk (in kilybytes) is displayed now. 

    But there's something strange... We formatted the disk double-sided and double-density, so according to the table in the beginning of this chapter, it has 720Kb of capacity. And since we copied a 32Kb file to it, there should be 688Kb available instead of the 679Kb returned by DSKF(). This lost space is used by these tables in the beginning of the disk that hold the information about the files stored on it.

Summary

These are the MSX-BASIC commands, instructions and functions seen today:

- CALL FORMAT

- * FILES

- * DSKF

- * PRINT

The ones marked with an asterisk are the ones that still have more parameters that we didn't see today.