| UNIX Implementation for MSX [UZIX] [Tech info] [Status] [TCP/IP] [WWW] [Shots] [Downloads] | Results of ethernet card prototype on UZIX |
| --- | --- |

Objectives:

    The primary purpose of this research is proving that a MSX can be connected to a network using an ethernet card. The secondary purpose is giving motivation to hardwaremakers to produce such a card. This is, until now, the first and only functional, tested and usable ethernet card for MSX machines.
    Adriano Cunha, the author, his company A&L Software, Ademir Carchano or CIEL, his company, will not produce such hardware. A&L Software will, certainly, create UZIX drivers for any ethernet card developed for MSX, since the required knowdledge was already acquired.

The hardware:

    For the ethernet hardware, the Rabbit 2000 TCP/IP Development Kit was used. This development kit contains a Rabbit 2000 microprocessor running at 18MHz and a RealTek RTL8019AS Ethernet Controller.

| The Rabbit DevKit used as ethernet card | A closer look in the chips |
| --- | --- |
| The RTL8019AS chip | The Rabbit 2000 microprocessor |

To make communication between MSX and the Rabbit DevKit, a former ATA IDE and former FlashROM cartridge from CIEL were modified. Any byte written or read on the interface memory area is sent or received by the Rabbit DevKit, using digital 4-bit sinchronous communication (each byte is split in two nibbles).

The MSX cartridge to communicate with the Rabbit DevKit

Finally, the Rabbit DevKit is plugged on a hub and on a MSX machine, being ready to be part of any network.

| The hub linking a MSX and a PC in a network | The MSX Turbo-R used for testing |
| --- | --- |

The software:

    The software needed to handle the ethernet card is divided between MSX and Rabbit DevKit.

- The software on Rabbit DevKit:

The Rabbit microprocessor is responsible for the communication with the RealTek RTL8019AS chip, since they are hardwired. The unique job of the Rabbit microprocessor is to send and receive data from I/O digital port (connected to MSX) and RealTek chip.
Absence of data (to Rabbit microprocessor or to MSX) is indicated by reading (at any side) value 0xFF (255) from I/O channel. When MSX sends a packet to Rabbit kit, the byte 0x80 (128) and the packet size is sent before the packet itself (that is sent in a burst). When MSX sends a value to be put in RTL8019AS registers, the byte 0x40 (64) is sent before the register number and its value; the register number is mapped and the value written directly in the RealTek chip by the Rabbit microprocessor.
When an ethernet packet arrives on the RTL8019AS, it's sent to MSX. The bytes 0x20 (32) and 0x10 (16) are used for handshaking. After this the packet size and the packet itself are sent (also in a burst).
For the software (and hardware) point of view, everything looks like if the RTL8019AS chip were hardwired to MSX.

- The software on MSX:

On MSX, the processing of data received or sent from/to the ethernet card is divided in two: the ethernet driver on UZIX kernel sends and receives ethernet packets; the TCP/IP module manages the encapsuled UDP, TCP, ICMP, IP and ARP packets.
To read data from the ethernet card, polling is used. A better implementation would use interruptions to signal that a packet arrived on the RTL8019AS chip. Data is sent/read from/to ethernet card in bursts. These two tasks are done by UZIX kernel.
The TCP/IP module decodes ethernet packets, replying to ARP packets and managing IP packets for UZIX applications.

Results:

     Here are a brief resume of the results of the tests with the ethernet card. The comparison with a PC were made against a Pentium III 500MHz computer with 128Mb RAM and 8GB HD running RedHat Linux 6.2.
     The MSX and the PC were connected to the same network by a hub. The network is a private network of NipNet, a local ISP that operates using a radio link.
     For the modem test, the same ISP, UOL, telephone line, access number and user were used. The modem used on MSX is a US Robotics Sportster 14.4kbps. On PC, the modem is a Genius GM56PCI-L 56kbps.
     The TCP/UDP transfer rates were measured using FudeBrowZer to download the UZIX webpage from http://uzix.sf.net and dividing the page size by the time it takes to be loaded. Ten measures were made to get the average transfer rate. It's not the maximum or absolute average transfer rate (that depends on time of day, network load, server load, etc), but can be used for relative comparison.
     All the tests were done during a week, from 23:30h to 00:30h.

| ping results (min/avg/max times in ms) |  |  |  |  |
| --- | --- | --- | --- | --- |
| MSX Turbo-R GT, R800 processor at 7MHz, 512kb RAM, CIEL ATA-IDE and 512kb MegaRAM |  |  |  |  |
| Host | MSX (ethernet) | MSX (14k4 modem) | PC (ethernet) | PC (56k modem) |
| localhost | 34/34/34 | 68/68/68 | 0/0/0 | 0/0/0 |
| network gateway | 34/68/102 | N/A | 16/47/108 | N/A |
| www.msx.org | 306/374/561 | 544/605/663 | 274/326/406 | 410/429/460 |
| dcc.unicamp.br | 68/90/136 | 238/321/391 | 34/68/127 | 140/158/200 |
| MSX Turbo-R GT, Z80 processor at 3.5MHz, 512kb RAM, CIEL ATA-IDE and 512kb MegaRAM |  |  |  |  |
| Host | MSX (ethernet) | MSX (14k4 modem) | PC (ethernet) | PC (56k modem) |
| localhost | 68/105/119 | 170/170/170 | 0/0/0 | 0/0/0 |
| network gateway | 119/196/391 | N/A | 16/29/45 | N/A |
| www.msx.org | 391/533/714 | 1173/1328/1496 | 311/375/452 | 440/457/501 |
| dcc.unicamp.br | 170/293/527 | 1190/1252/1309 | 47/97/164 | 161/173/219 |
| MSX 2+ Turbo CIEL, Z80 processor at 7MHz, 1Mb RAM, Sunrise ATA-IDE |  |  |  |  |
| Host | MSX (ethernet) | MSX (14k4 modem) | PC (ethernet) | PC (56k modem) |
| localhost | 51/51/51 | 119/119/119 | 0/0/0 | 0/0/0 |
| network gateway | 51/97/153 | N/A | 15/33/21 | N/A |
| www.msx.org | 408/566/969 | 714/778/1207 | 363/543/897 | 399/445/478 |
| dcc.unicamp.br | 102/206/816 | 425/491/629 | 43/138/254 | 153/188/205 |
| TCP/UDP transfer rate (average value, in kb/s) |  |  |  |  |
| Machine |  |  |  | Transfer rate |
| MSX Turbo-R (Z80 mode, 3.5MHz), 14k4 modem |  |  |  | 0.16 |
| MSX 2+ Turbo CIEL (7MHz), 14k4 modem |  |  |  | 0.46 |
| MSX Turbo-R (R800 mode, 7MHz), 14k4 modem |  |  |  | 0.82 |
| MSX Turbo-R (Z80 mode, 3.5MHz), ethernet |  |  |  | 1.59 |
| MSX 2+ Turbo CIEL (7MHz), ethernet |  |  |  | 2.58 |
| MSX Turbo-R (R800 mode, 7MHz), ethernet |  |  |  | 3.99 |
| Pentium III 500MHz, 56k modem |  |  |  | 4.82 |
| Pentium III 500MHz, ethernet |  |  |  | 69.28 |

Conclusions and proposals:

     As already concluded in RS232 speed comparison under UZIX, the bottleneck for high-speed communication (serial, paralel, ethernet, etc) using MSX is the CPU power.
     Even if spetacular transfer rates (like 10 or 20kb/s) couldn't be achieved with this ethernet card prototype for MSX, it was proved that this computer can really be connected to an ethernet network, being active part of it.
     Probably, any ethernet card produced for MSX can't reach transfer rates too much higher than the ones achieved with this prototype, if the hardware design follows the one used in Sunrise's RS232 (incoming bytes generate interrupts and MSX reads each one from 16550 UART). Maybe, using the approach of Harukaze RS232 (an internal 32kb buffer filled by the 16550 UART itself), higher speeds can be achieved, since reading an entire packet will be faster (and packet loss due to a missed byte will not occur).
     Also, checking achieved transfer rates for different values of MTU for TCP packets should lead to the best value for ethernet communication, resulting in speed gain. This performance research was not made (yet).
     Radical optimization alternatives such as rewriting UZIX TCP/IP module or UZIX kernel entirely in assembly language are not considered here. Also, using the ethernet card prototype on monotask and TCP/IP-less operating systems (such as MSXDOS) was disconsidered.

Acknowledgements:

     Special thanks go to Ademir Carchano for his precious help with hardware subjects. Also, Ademir is the owner of the Rabbit DevKit used in this prototype.
     Also thanks to Werner Kai, from Core Club, for lendging his WebCam to take the photos for this page and his help with hardware questions.

     This page was put online on 06/12/2001. Last update was on 06/12/2001 by Adriano Cunha.